# 13P2 Hook Panel Specifications

PART 1 - GENERAL

- DESCRIPTION: Furnish and install wall mounted hook panels.

- RELATED WORK: (Refer to Section of Specs which applies to wall blocking.)

PART 2 - PRODUCTS

- MANUFACTURER: 13P2 series hook panel by EMCO Specialty Products, Inc., Kansas City, KS.

- MATERIALS: Panel shall be of solid (walnut or oak) 5/8”x 5 5/8”x 17” hardwood and drilled for standard number of hooks. The finish is to be (specify walnut, medium oak, or light oak).

- 13P2 panels shall have two P1 hooks (specify finish either silver or black) and one Y10 slotted hook to accommodate two hangers. All hooks to be installed at the factory.

- ACCESSORIES: One EMCO hanger No.13 ball top hanger shall be provided for each available slot in the center hook. Hanger to match panel finish.

PART 3 - EXECUTION

INSTALLATION:

  - Install wall blocking.

  - Install concealed fastener mounting plate to wall using “fasteners by others” appropriate for wall type.

  - Install panel at height specified by architect.

  - Check to make sure it is level.

  - Remove adhesive backing strip from back of panel.

  - Slide panel concealed fastener over wall mounting plate.

  - Install hangers in hook slots.

  - Wipe down panel to remove dust.

  - Remove all trash and debris.